# Strength of wire, joints, poles and motor: Agrovoltaic Canopy for Protection and Sustainable Farming

Will it give good output? Yes, for the final design. At 140 km/h (panels held) every rope, tube, bearing and pole is within its limit. The first parts list had weak items, now changed: M16 turnbuckles, undersized anchor blocks, a 4 mm drive wire (too tight) and a 3 kN winch (limit at about 45 km/h wind), and no wind sensor or stow arm. The final design adds a cup anemometer and a 45 degree offset lever arm for the 90 degree storm stow.

| Part | What it does | Load on it | What it can carry | Load / capacity | Verdict | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier rope SS316 16 mm 7x19 | holds the tubes (5 m span) | 26.8 kN at 140 km/h wind | 72.7 kN (145.5 breaking / 2) | 0.37 | GOOD | Breaking load from the supplier listing. |
| Stay rope SS316 20 mm | keeps the columns from being pulled in | 36.1 kN at 140 km/h wind | 114 kN (indicative) | 0.32 | GOOD |  |
| Rope end fittings (swaged) | rope to turnbuckle and column | 36.1 kN | about 90 % of rope breaking load | 0.2 (est.) | GOOD if swaged | Factory swages, not U-clips. |
| Turnbuckle M24 SS316 (changed from M16) | tensions the stay | 36.1 kN | select one rated >= 72 kN breaking | 0.5 | GOOD (after change) | M16 working loads are only a few kN, so it was too weak. |
| Pivot tube CHS 114.3 x 4, S355 | turns with the panels | 10.0 kNm | 13.11 kNm | 0.76 | GOOD | Tube length 4.7 m between carrier lines. Do not lengthen. |
| Bearing block (rotating joint) 50 mm | lets the tube rotate under the rope | 8.5 kN down / 7.9 kN up | about 23 kN static (typical 50 mm block) | 0.37 | GOOD | Typical catalogue value; confirm with the chosen bearing. |
| Column RHS 200 x 200 x 6 | holds everything up | 8.5 kN + 7.9 kN uplift | 802 kN buckling (0.6 Pcr) | 0.05 | VERY STRONG | A smaller column would also work. |
| Column pad 0.9 x 0.9 x 0.6 m | ties the column to the ground | 7.9 kN uplift | pad weight 11.4 kN | 0.69 | OK | Needs a soil test. |
| Ground anchor (changed) | holds the stay down | 25.6 kN uplift (x 1.5 = 38 kN) | buried concrete 1.63 m3 each, or screw/rock anchor >= 38 kN | <= 1.0 by design | GOOD (after change) | The old block 1.0 x 0.8 x 0.35 m resisted only 6.6 kN. |
| Drive wire SS316 6 mm 7x19 (changed from 4 mm) | pulls the lever arms | 0.4 / 2.1 / 4.1 kN (calm / 36 km/h / 54 km/h wind) | 10.2 kN (20.4 breaking / 2) | 0.04 / 0.20 / 0.41 | GOOD to 54 km/h | At 60 degrees tilt, the worst angle, panels held. The old 4 mm wire would be at 0.91 of its limit at 54 km/h: too tight. |
| Cup anemometer + mast (new) | measures wind for the storm stow | 15 m/s trigger (54 km/h) | typical cup anemometer 0 to 40 m/s | < 0.5 | GOOD | The stow moves the panels edge-on to 90 degrees; resume below 12 m/s. |
| Lever arm SS316 (0.44 m, 45 deg offset) | turns the tube | 439 Nm per row at 36 km/h | arm plate: well above this | below 0.5 (est.) | GOOD | Check the weld and the clamp to the tube. |
| Gearmotor winch 10 kN (changed from 3 kN) | winds the drive wire | 0.4 / 2.1 / 4.1 kN | 10 kN | 0.04 / 0.21 / 0.41 | GOOD to 54 km/h | The old 3 kN winch reaches its limit at about 45 km/h wind and is at 1.38 of its limit at 54 km/h. |

# Motor and drive numbers

Each row of 4 panels weighs about 176 kg; an assumed 5 cm centre-of-mass offset gives about 86 Nm per row.

Wind torque per row = 0.6 V^2 x area (10.3 m2) x offset (0.57 m), flat-plate coefficient 1.0 (conservative).

The lever arm is fixed with a 45 degree offset, so the useful lever is 0.44 m x cos(tilt - 45 deg): 0.43 m at 60 degrees and still 0.31 m at the 90 degree stow. Wire force = torque of 2 rows / 0.43 m: 0.4 kN calm, 2.1 kN at 36 km/h, 4.1 kN at 54 km/h.

Speed: 15 to 60 degrees needs 0.27 m of wire in 45 s (6 mm/s). Mechanical power is only about 30 W at 5 kN, so a 0.25 kW worm-gear motor is ample.

Wind stow: the anemometer trips the stow at 15 m/s (54 km/h) and releases below 12 m/s; edge-on panels (90 degrees) carry very little wind torque.

# Honest limits

Screening-level hand calculation (IS 875 style), not a certified design.

Bearing, turnbuckle, winch and lever-arm capacities are typical catalogue values or estimates; confirm with the datasheets of the parts you buy.

Wind torque uses a flat-plate coefficient of 1.0 and a centre of pressure a quarter of the panel width from the axis.