Review 0: one-page write-up for supervisor approval

Project: Wire-driven agrivoltaic canopy with sensor-driven tilt control (SRMC), TCE ground, Madurai

Student: ____________________     Register no.: ______________     Guide: ____________________     Date: ____________

1. Objective

Choose the panel tilt angle that gives the most solar energy while the crop below still receives the light it needs. One controller uses: pyranometer (energy), PAR (crop light), soil moisture (irrigation and stress), DHT11 (temperature, humidity, VPD stress), rain sensor (rain-harvest slope) and wind (safety stow).

2. What will be demonstrated in Review 0

Real sensors on the prototype: DHT11, soil moisture and rain sensor connected to an ESP8266 NodeMCU (ESP32 as backup); data sent over USB serial to MATLAB every second.

Virtual (simulated) pyranometer, PAR and wind in MATLAB, because these sensors are not yet available. They are computed from the validated sun and canopy-shadow model and are labelled SIM on screen.

Live decision: rain detected -> panels to a 3 degree slope (harvest); wind >= 15 m/s -> stow at 90 degrees; night -> park; otherwise an optimiser picks the angle (15 to 60 degrees, 0.5 degree steps) that balances energy and crop light. The tilt is shown on the MATLAB screen only (no actuator).

3. Evidence that will be shown

16 software-in-the-loop tests of the decision logic, all passing; physics identical to the earlier validated model (PVGIS comparison +5.0 %, Blender ray-tracing check 0.010).

Bench measurements on the day: DHT11 against a reference thermometer, soil dry/wet calibration, rain detection time, end-to-end response, with photos and serial/MATLAB screenshots and a saved CSV log.

Day simulation: the controller keeps about 75 % of the best energy and 89 % of the best crop light on a clear day; pure sun tracking keeps 100 % energy but 27 % of the crop light.

4. Limits stated openly

Pyranometer, PAR and wind are simulated; tilt is not mechanised; DHT11 accuracy is +-2 C and +-5 % RH; models are screening-level and validated against PVGIS and Blender, not against field data. Prices and income figures elsewhere in the project are assumptions.

5. Resources

NodeMCU ESP8266 (ESP32 spare), DHT11, soil moisture module, rain sensor module, laptop with MATLAB and Arduino IDE.

| Student signature | Supervisor signature and date | HDEE |
| --- | --- | --- |